Configuration

Basic Configuration Steps: -

Basic settings to all devices plus ssh on the 2 routers and 17 switches.

- Subnetting and IP addressing

- OSPF on the routers and 13 switches.

- Static IP address to server Room devices.

- DHCP server device configurations.

- Inter-VLAN routing on the 13 switches plus IP DHCP helper addresses.

- Wireless network configurations.

- PAT + Access Control List

- Verifying and testing configurations.

Cafeteria CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname Cafeteria-SW

Cafeteria-SW(config)#banner motd #NO Unauthorised access!!!#

Cafeteria-SW(config)#no ip domain lookup

Cafeteria-SW(config)#line console 0

Cafeteria-SW(config-line)#passw cafeteria1234

Cafeteria-SW(config-line)#login

Cafeteria-SW(config-line)#ex

% Ambiguous command: "ex"

Cafeteria-SW(config-line)#exit

Cafeteria-SW(config)#enable passw cafeteria1234

Cafeteria-SW(config)#service password-encryption

Cafeteria-SW(config)#ex

Cafeteria-SW#

%SYS-5-CONFIG_I: Configured from console by console

Cafeteria-SW#wr

Building configuration...

[OK]

Cafeteria-SW#

Reception & Visitor Lounge CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname Reception-SW

Reception-SW(config)#banner motd #NO Unauthorised access!!!#

Reception-SW(config)#no ip domain lookup

Reception-SW(config)#line console 0

Reception-SW(config-line)#passw reception1234

Reception-SW(config-line)#login

Reception-SW(config-line)#ex

% Ambiguous command: "ex"

Reception-SW(config-line)#exit

Reception-SW(config)#enable passw reception1234

Reception-SW(config)#service password-encryption

Reception-SW(config)#ex

Reception-SW#

%SYS-5-CONFIG_I: Configured from console by console

Reception-SW#wr

Building configuration...

[OK]

Reception-SW#

Industrial Devices Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname ID-SW

ID-SW(config)#line console 0

ID-SW(config-line)#password cisco

ID-SW(config-line)#login

ID-SW(config-line)#exit

ID-SW(config)#enable password cisco

ID-SW(config)#no ip domain-lookup

ID-SW(config)#banner motd #NO Unauthorised Access! ! !#

ID-SW(config)#service password-encryption

ID-SW(config)#do wr

Building configuration...

[OK]

ID-SW(config)#

ID-SW(config)#

Manufacturing Production Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname MPD-SW

MPD-SW(config)#line console 0

MPD-SW(config-line)#password cisco

MPD-SW(config-line)#login

MPD-SW(config-line)#exit

MPD-SW(config)#enable password cisco

MPD-SW(config)#no ip domain-lookup

MPD-SW(config)#banner motd #NO Unauthorised Access! ! !#

MPD-SW(config)#service password-encryption

MPD-SW(config)#do wr

Building configuration...

[OK]

MPD-SW(config)#

Engineering Workstation Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname EWD-SW

EWD-SW(config)#line console 0

EWD-SW(config-line)#password cisco

EWD-SW(config-line)#login

EWD-SW(config-line)#exit

EWD-SW(config)#enable password cisco

EWD-SW(config)#no ip domain-lookup

EWD-SW(config)#banner motd #NO Unauthorised Access! ! !#

EWD-SW(config)#service password-encryption

EWD-SW(config)#do wr

Building configuration...

[OK]

EWD-SW(config)#

Design & Maintenance Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname DMD-SW

DMD-SW(config)#line console 0

DMD-SW(config-line)#password cisco

DMD-SW(config-line)#login

DMD-SW(config-line)#exit

DMD-SW(config)#enable password cisco

DMD-SW(config)#no ip domain-lookup

DMD-SW(config)#banner motd #NO Unauthorised Access! ! !#

DMD-SW(config)#service password-encryption

DMD-SW(config)#do wr

Building configuration...

[OK]

DMD-SW(config)#

DMD-SW(config)#

Inventory Management Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname IMD-SW

IMD-SW(config)#line console 0

IMD-SW(config-line)#password cisco

IMD-SW(config-line)#login

IMD-SW(config-line)#exit

IMD-SW(config)#enable password cisco

IMD-SW(config)#no ip domain-lookup

IMD-SW(config)#banner motd #NO Unauthorised Access! ! !#

IMD-SW(config)#service password-encryption

IMD-SW(config)#do wr

Building configuration...

[OK]

IMD-SW(config)#

IMD-SW(config)#

Backup Storage Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname BSD-SW

BSD-SW(config)#line console 0

BSD-SW(config-line)#password cisco

BSD-SW(config-line)#login

BSD-SW(config-line)#exit

BSD-SW(config)#enable password cisco

BSD-SW(config)#no ip domain-lookup

BSD-SW(config)#banner motd #NO Unauthorised Access! ! !#

BSD-SW(config)#service password-encryptio

BSD-SW(config)#do wr

Building configuration...

[OK]

BSD-SW(config)#

Logistics Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname LD-SW

LD-SW(config)#line console 0

LD-SW(config-line)#password cisco

LD-SW(config-line)#login

LD-SW(config-line)#exit

LD-SW(config)#enable password cisco

LD-SW(config)#no ip domain-lookup

LD-SW(config)#banner motd #NO Unauthorised Access! ! !#

LD-SW(config)#service password-encryption

LD-SW(config)#do wr

Building configuration...

[OK]

LD-SW(config)#

Human Resources Dept CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname HRD-SW

HRD-SW(config)#line console 0

HRD-SW(config-line)#password cisco

HRD-SW(config-line)#login

HRD-SW(config-line)#exit

HRD-SW(config)#enable password cisco

HRD-SW(config)#no ip domain-lookup

HRD-SW(config)#banner motd #NO Unauthorised Access! ! !#

HRD-SW(config)#service password-encryption

HRD-SW(config)#do wr

Building configuration...

[OK]

HRD-SW(config)#
Finance & Accounts Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname FAD-SW

FAD-SW(config)#line console 0

FAD-SW(config-line)#password cisco

FAD-SW(config-line)#login

FAD-SW(config-line)#exit

FAD-SW(config)#enable password cisco

FAD-SW(config)#no ip domain-lookup

FAD-SW(config)#banner motd #NO Unauthorised Access! ! !#

FAD-SW(config)#service password-encryption

FAD-SW(config)#do wr

Building configuration...

[OK]

FAD-SW(config)#
Sales & Marketing Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SMD-SW

SMD-SW(config)#line console 0

SMD-SW(config-line)#password cisco

SMD-SW(config-line)#login

SMD-SW(config-line)#exit

SMD-SW(config)#enable password cisco

SMD-SW(config)#no ip domain-lookup

SMD-SW(config)#banner motd #NO Unauthorised Access! ! !#

SMD-SW(config)#service password-encryption

SMD-SW(config)#do wr

Building configuration...

[OK]

SMD-SW(config)#
Administrator & Public Relation Dept CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname APR-SW

APR-SW(config)#line console 0

APR-SW(config-line)#password cisco

APR-SW(config-line)#login

APR-SW(config-line)#exit

APR-SW(config)#enable password cisco

APR-SW(config)#no ip domain-lookup

APR-SW(config)#banner motd #NO Unauthorised Access! ! !#

APR-SW(config)#service password-encryption

APR-SW(config)#do wr

Building configuration...

[OK]

APR-SW(config)#
IT Dept CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname ITD-SW

ITD-SW(config)#line console 0

ITD-SW(config-line)#password cisco

ITD-SW(config-line)#login

ITD-SW(config-line)#exit

ITD-SW(config)#enable password cisco

ITD-SW(config)#no ip domain-lookup

ITD-SW(config)#banner motd #NO Unauthorised Access! ! !#

ITD-SW(config)#service password-encryption

ITD-SW(config)#do wr

Building configuration...

[OK]

ITD-SW(config)#
Vendor Portals & Communication Dept CLI Commands: -

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname VPC-SW

VPC-SW(config)#line console 0

VPC-SW(config-line)#password cisco

VPC-SW(config-line)#login

VPC-SW(config-line)#exit

VPC-SW(config)#enable password cisco

VPC-SW(config)#no ip domain-lookup

VPC-SW(config)#banner motd #NO Unauthorised Access! ! !#

VPC-SW(config)#service password-encryption

VPC-SW(config)#do wr

Building configuration...

[OK]

VPC-SW(config)#
Security Dept CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SD-SW

SD-SW(config)#line console 0

SD-SW(config-line)#password cisco

SD-SW(config-line)#login

SD-SW(config-line)#exit

SD-SW(config)#enable password cisco

SD-SW(config)#no ip domain-lookup

SD-SW(config)#banner motd #NO Unauthorised Access! ! !#

SD-SW(config)#service password-encryption

SD-SW(config)#do wr

Building configuration...

[OK]

SD-SW(config)#
Server Room CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SR-SW

SR-SW(config)#line console 0

SR-SW(config-line)#password cisco

SR-SW(config-line)#login

SR-SW(config-line)#exit

SR-SW(config)#enable password cisco

SR-SW(config)#no ip domain-lookup

SR-SW(config)#banner motd #NO Unauthorised Access! ! !#

SR-SW(config)#service password-encryption

SR-SW(config)#do wr

Building configuration...

[OK]

SR-SW(config)#
Multilayer Switch 1 CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname MS1-SW

MS1-SW(config)#line console 0

MS1-SW(config-line)#password cisco

MS1-SW(config-line)#login

MS1-SW(config-line)#exit

MS1-SW(config)#enable password cisco

MS1-SW(config)#no ip domain-lookup

MS1-SW(config)#banner motd #NO Unauthorised Access! ! !#

MS1-SW(config)#service password-encryption

MS1-SW(config)#do wr

Building configuration...

Compressed configuration from 7383 bytes to 3601 bytes[OK]

[OK]

MS1-SW(config)#ip domain name cisco.net

MS1-SW(config)#username admin password cisco

MS1-SW(config)#crypto key generate rsa

The name for the keys will be: MS1-SW.cisco.net

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

MS1-SW(config)#line vty 15

*Mar 1 0:38:48.550: %SSH-5-ENABLED: SSH 1.99 has been enabled

MS1-SW(config-line)#login local

MS1-SW(config-line)#transport input ssh

MS1-SW(config-line)#exit

MS1-SW(config)#do wr

Building configuration...

Compressed configuration from 7383 bytes to 3601 bytes[OK]

[OK]

MS1-SW(config)#
ML1-SW(config)#ip ssh version 2

Multilayer Switch 2 CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname ML2-SW

ML2-SW(config)#line console 0

ML2-SW(config-line)#password cisco

ML2-SW(config-line)#login

ML2-SW(config-line)#exit

ML2-SW(config)#enable password cisco

ML2-SW(config)#no ip domain-lookup

ML2-SW(config)#banner motd #NO Unauthorised Access! ! !#

ML2-SW(config)#service password-encryption

ML2-SW(config)#do wr

Building configuration...

Compressed configuration from 7383 bytes to 3601 bytes[OK]

[OK]

ML2-SW(config)#ip domain name cisco.net

ML2-SW(config)#username admin password cisco

ML2-SW(config)#crypto key generate rsa

The name for the keys will be: ML2-SW.cisco.net

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

ML2-SW(config)#line vty 15

*Mar 1 0:38:48.550: %SSH-5-ENABLED: SSH 1.99 has been enabled

ML2-SW(config-line)#login local

ML2-SW(config-line)#transport input ssh

ML2-SW(config-line)#exit

ML2-SW(config)#do wr

Building configuration...

Compressed configuration from 7383 bytes to 3601 bytes[OK]

[OK]

ML2-SW(config)#
ML2-SW(config)#ip ssh version 2

Multilayer Switch 3 CLI Commands: -
Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname ML3-SW

ML3-SW(config)#line console 0

ML3-SW(config-line)#password cisco

ML3-SW(config-line)#login

ML3-SW(config-line)#exit

ML3-SW(config)#enable password cisco

ML3-SW(config)#no ip domain-lookup

ML3-SW(config)#banner motd #NO Unauthorised Access! ! !#

ML3-SW(config)#service password-encryption

ML3-SW(config)#do wr

Building configuration...

Compressed configuration from 7383 bytes to 3601 bytes[OK]

[OK]

ML3-SW(config)#ip domain name cisco.net

ML3-SW(config)#username admin password cisco

ML3-SW(config)#crypto key generate rsa

The name for the keys will be: ML3-SW.cisco.net

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

ML3-SW(config)#line vty 15

*Mar 1 0:38:48.550: %SSH-5-ENABLED: SSH 1.99 has been enabled

ML3-SW(config-line)#login local

ML3-SW(config-line)#transport input ssh

ML3-SW(config-line)#exit

ML3-SW(config)#do wr

Building configuration...

Compressed configuration from 7383 bytes to 3601 bytes[OK]

[OK]

ML3-SW(config)#
ML3-SW(config)#ip ssh version 2

Core Router 1 CLI Commands: -
Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname Core-R1

Core-R1(config)#line console 0

Core-R1(config-line)#password cisco

Core-R1(config-line)#login

Core-R1(config-line)#exit

Core-R1(config)#enable password cisco

Core-R1(config)#no ip domain-lookup

Core-R1(config)#banner motd #NO Unauthorised Access! ! !#

Core-R1(config)#service password-encryption

Core-R1(config)#do wr

Building configuration...

[OK]

Core-R1(config)#ip domain name cisco.net

Core-R1(config)#username admin password cisco

Core-R1(config)#crypto key generate rsa

The name for the keys will be: Core-R1.cisco.net

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

Core-R1(config)#line vty 015

*Mar 1 1:24:50.156: %SSH-5-ENABLED: SSH 1.99 has been enabled

Core-R1(config-line)#login local

Core-R1(config-line)#transport input ssh

Core-R1(config-line)#exit

Core-R1(config)#do wr

Building configuration...

[OK]

Core-R1(config)#
Core-R1(config)#ip ssh version 2

Core Router 2 CLI Commands: -
Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname Core-R2

Core-R2(config)#line console 0

Core-R2(config-line)#password cisco

Core-R2(config-line)#login

Core-R2(config-line)#exit

Core-R2(config)#enable password cisco

Core-R2(config)#no ip domain-lookup

Core-R2(config)#banner motd #NO Unauthorised Access! ! !#

Core-R2(config)#service password-encryption

Core-R2(config)#do wr

Building configuration...

[OK]

Core-R2(config)#ip domain name cisco.net

Core-R2(config)#username admin password cisco

Core-R2(config)#crypto key generate rsa

The name for the keys will be: Core-R2.cisco.net

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

Core-R2(config)#line vty 015

*Mar 1 1:26:23.983: %SSH-5-ENABLED: SSH 1.99 has been enabled

Core-R2(config-line)#login local

Core-R2(config-line)#transport input ssh

Core-R2(config-line)#exit

Core-R2(config)#do wr

Building configuration...

[OK]

Core-R2(config)#
Core-R2(config)#ip ssh version 2

VLANs assignment plus all access and trunk ports on 12 and 13 switches.

Cafeteria CLI Commands: -
Cafeteria-SW(config)#int range fa0/1-3

Cafeteria-SW(config-if-range)#switch mode trunk

Cafeteria-SW(config-if-range)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Cafeteria-SW(config-if-range)#exi

Cafeteria-SW(config)# int range fa0/4-24

Cafeteria-SW(config-if-range)#switchport mode access

Cafeteria-SW(config-if-range)#switchport access vlan 10

% Access VLAN does not exist. Creating vlan 10

Cafeteria-SW(config-if-range)#ex

Cafeteria-SW(config)#vlan 10

Cafeteria-SW(config-vlan)#name Cafeteria

Cafeteria-SW(config-vlan)#ex

Cafeteria-SW(config)#do wr

Building configuration...

[OK]

Cafeteria-SW(config)#

Reception CLI Commands: -
Reception-SW(config)#int range fa0/1-3

Reception-SW(config-if-range)#switchport mode trunk

Reception-SW(config-if-range)#exit

Reception-SW(config)#

Reception-SW(config)#vlan 61

Reception-SW(config-vlan)#name Reception

Reception-SW(config-vlan)#exit

Reception-SW(config)#

Reception-SW(config)#int range fa0/4-24

Reception-SW(config-if-range)#switchport mode access

Reception-SW(config-if-range)#switchport access vlan 61

Reception-SW(config-if-range)#exit

Reception-SW(config)#

Reception-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Industrial Devices Dept CLI Commands: -
ID-SW(config)#int range fa0/1-3

ID-SW(config-if-range)#switchport mode trunk

ID-SW(config-if-range)#exit

ID-SW(config)#

ID-SW(config)#vlan 60

ID-SW(config-vlan)#name ID

ID-SW(config-vlan)#exit

ID-SW(config)#

ID-SW(config)#int range fa0/4-24

ID-SW(config-if-range)#switchport mode access

ID-SW(config-if-range)#switchport access vlan 60

ID-SW(config-if-range)#exit

ID-SW(config)#

ID-SW(config)#do wr

Building configuration...

[OK]

ID-SW(config)#
Manufacturing Production Lines Dept CLI Commands: -
MPD-SW(config)#int range fa0/1-3

MPD-SW(config-if-range)#switchport mode trunk

MPD-SW(config-if-range)#exit

MPD-SW(config)#

MPD-SW(config)#vlan 11

MPD-SW(config-vlan)#name MPLD

MPD-SW(config-vlan)#exit

MPD-SW(config)#

MPD-SW(config)#int range fa0/4-24

MPD-SW(config-if-range)#switchport mode access

MPD-SW(config-if-range)#switchport access vlan 11

MPD-SW(config-if-range)#exit

MPD-SW(config)#

MPD-SW(config)#do w

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

MPD-SW(config)#

Engineering Workstations Dept CLI Commands: -

EWD-SW(config-if-range)#exit

EWD-SW(config)#

EWD-SW(config)#vlan 20

EWD-SW(config-vlan)#name EWD

EWD-SW(config-vlan)#exit

EWD-SW(config)#

EWD-SW(config)#int range fa0/4-24

EWD-SW(config-if-range)#switchport mode access

EWD-SW(config-if-range)#switchport access vlan 20

EWD-SW(config-if-range)#exit

EWD-SW(config)#

EWD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

EWD-SW(config)#

Design & Maintenance Dept CLI Commands: -

DMD-SW(config)#int range fa0/1-3

DMD-SW(config-if-range)#switchport mode trunk

DMD-SW(config-if-range)#exit

DMD-SW(config)#

DMD-SW(config)#vlan 21

DMD-SW(config-vlan)#name DMD

DMD-SW(config-vlan)#exit

DMD-SW(config)#

DMD-SW(config)#int range fa0/4-24

DMD-SW(config-if-range)#switchport mode access

DMD-SW(config-if-range)#switchport access vlan 21

DMD-SW(config-if-range)#exit

DMD-SW(config)#

DMD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

DMD-SW(config)#

Inventory & Management Dept CLI Commands: -

IMD-SW(config)#int range fa0/1-3

IMD-SW(config-if-range)#switchport mode trunk

IMD-SW(config-if-range)#exit

IMD-SW(config)#

IMD-SW(config)#vlan 40

IMD-SW(config-vlan)#name DMD

IMD-SW(config-vlan)#exit

IMD-SW(config)#

IMD-SW(config)#int range fa0/4-24

IMD-SW(config-if-range)#switchport mode access

IMD-SW(config-if-range)#switchport access vlan 40

IMD-SW(config-if-range)#exit

IMD-SW(config)#

IMD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/5 (40), with ML1-SW GigabitEthernet1/0/12 (1).

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/7 (40), with ML2-SW GigabitEthernet1/0/11 (1).

Building configuration...

[OK]

IMD-SW(config)#

Backup Storage Dept CLI Commands: -

BSD-SW(config)#int range fa0/1-3

BSD-SW(config-if-range)#switchport mode trunk

BSD-SW(config-if-range)#exit

BSD-SW(config)#

BSD-SW(config)#vlan 42

BSD-SW(config-vlan)#name BSD

BSD-SW(config-vlan)#exit

BSD-SW(config)#

BSD-SW(config)#int range fa0/4-24

BSD-SW(config-if-range)#switchport mode access

BSD-SW(config-if-range)#switchport access vlan 42

BSD-SW(config-if-range)#exit

BSD-SW(config)#

BSD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

BSD-SW(config)#

Logistics Dept CLI Commands: -

LD-SW(config)#int range fa0/1-3

LD-SW(config-if-range)#switchport mode trunk

LD-SW(config-if-range)#exit

LD-SW(config)#

LD-SW(config)#vlan 43

LD-SW(config-vlan)#name LD

LD-SW(config-vlan)#exit

LD-SW(config)#

LD-SW(config)#int range fa0/4-24

LD-SW(config-if-range)#switchport mode access

LD-SW(config-if-range)#switchport access vlan 43

LD-SW(config-if-range)#exit

LD-SW(config)#

LD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

LD-SW(config)#

Human Resources Dept CLI Commands: -

HRD-SW(config)#int range fa0/1-3

HRD-SW(config-if-range)#switchport mode trunk

HRD-SW(config-if-range)#exit

HRD-SW(config)#

HRD-SW(config)#vlan 31

HRD-SW(config-vlan)#name HR

HRD-SW(config-vlan)#exit

HRD-SW(config)#

HRD-SW(config)#int range fa0/4-24

HRD-SW(config-if-range)#switchport mode access

HRD-SW(config-if-range)#switchport access vlan 31

HRD-SW(config-if-range)#exit

HRD-SW(config)#

HRD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

HRD-SW(config)#

Finance & Accounts Dept CLI Commands: -

FAD-SW(config)#int range fa0/1-3

FAD-SW(config-if-range)#switchport mode trunk

FAD-SW(config-if-range)#exit

FAD-SW(config)#

FAD-SW(config)#vlan 32

FAD-SW(config-vlan)#name FAD

FAD-SW(config-vlan)#exit

FAD-SW(config)#

FAD-SW(config)#int range fa0/4-24

FAD-SW(config-if-range)#switchport mode access

FAD-SW(config-if-range)#switchport access vlan 32

FAD-SW(config-if-range)#exit

FAD-SW(config)#

FAD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

FAD-SW(config)#

Sales & Marketing Dept CLI Commands: -

SMD-SW(config)#int range fa0/1-3

SMD-SW(config-if-range)#switchport mode trunk

SMD-SW(config-if-range)#exit

SMD-SW(config)#

SMD-SW(config)#vlan 30

SMD-SW(config-vlan)#name SMD

SMD-SW(config-vlan)#exit

SMD-SW(config)#

SMD-SW(config)#int range fa0/4-24

SMD-SW(config-if-range)#switchport mode access

SMD-SW(config-if-range)#switchport access vlan 30

SMD-SW(config-if-range)#exit

SMD-SW(config)#

SMD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

SMD-SW(config)#

Administration & Public Relations Dept CLI Commands: -

APR-SW(config)#int range fa0/1-3

APR-SW(config-if-range)#switchport mode trunk

APR-SW(config-if-range)#exit

APR-SW(config)#

APR-SW(config)#vlan 33

APR-SW(config-vlan)#name APRD

APR-SW(config-vlan)#exit

APR-SW(config)#

APR-SW(config)#int range fa0/4-24

APR-SW(config-if-range)#switchport mode access

APR-SW(config-if-range)#switchport access vlan 33

APR-SW(config-if-range)#exit

APR-SW(config)#

APR-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

APR-SW(config)#

IT Dept CLI Commands: -

ITD-SW(config)#int range fa0/1-3

ITD-SW(config-if-range)#switchport mode trunk

ITD-SW(config-if-range)#exit

ITD-SW(config)#

ITD-SW(config)#vlan 34

ITD-SW(config-vlan)#name ITD

ITD-SW(config-vlan)#exit

ITD-SW(config)#

ITD-SW(config)#int range fa0/4-24

ITD-SW(config-if-range)#switchport mode access

ITD-SW(config-if-range)#switchport access vlan 34

ITD-SW(config-if-range)#exit

ITD-SW(config)#

ITD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

ITD-SW(config)#

Vendor Portals & Communication System Dept CLI Commands: -

VPC-SW(config)#int range fa0/1-3

VPC-SW(config-if-range)#switchport mode trunk

VPC-SW(config-if-range)#exit

VPC-SW(config)#

VPC-SW(config)#vlan 50

VPC-SW(config-vlan)#name VPCD

VPC-SW(config-vlan)#exit

VPC-SW(config)#

VPC-SW(config)#int range fa0/4-24

VPC-SW(config-if-range)#switchport mode access

VPC-SW(config-if-range)#switchport access vlan 50

VPC-SW(config-if-range)#exit

VPC-SW(config)#

VPC-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

VPC-SW(config)#
Security Dept CLI Commands: -

SD-SW(config)#int range fa0/1-3

SD-SW(config-if-range)#switchport mode trunk

SD-SW(config-if-range)#exit

SD-SW(config)#

SD-SW(config)#vlan 52

SD-SW(config-vlan)#name SD

SD-SW(config-vlan)#exit

SD-SW(config)#

SD-SW(config)#int range fa0/4-24

SD-SW(config-if-range)#switchport mode access

SD-SW(config-if-range)#switchport access vlan 52

SD-SW(config-if-range)#exit

SD-SW(config)#

SD-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

SD-SW(config)
Server Room CLI Commands: -

SR-SW(config)#int range fa0/1-3

SR-SW(config-if-range)#switchport mode trunk

SR-SW(config-if-range)#exit

SR-SW(config)#

SR-SW(config)#vlan 51

SR-SW(config-vlan)#name SR

SR-SW(config-vlan)#exit

SR-SW(config)#

SR-SW(config)#int range fa0/4-24

SR-SW(config-if-range)#switchport mode access

SR-SW(config-if-range)#switchport access vlan 51

SR-SW(config-if-range)#exit

SR-SW(config)#

SR-SW(config)#do wr

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

Building configuration...

[OK]

SR-SW(config)#

Switchport security to all departments.

Cafeteria CLI Commands: -

Cafeteria-SW(config)#int range fa0/4-24

Cafeteria-SW(config-if-range)#switchport port-security maximum 1

Cafeteria-SW(config-if-range)#switchport port-security mac-address sticky

Cafeteria-SW(config-if-range)#switchport port-security violation shutdown

Cafeteria-SW(config-if-range)#ex

Cafeteria-SW(config)#do wr

Building configuration...

[OK]
Receptions CLI Commands: -
Reception-SW(config)#int range fa0/4-24

Reception-SW(config-if-range)#switchport port-security maximum 1

Reception-SW(config-if-range)#switchport port-security mac-address sticky

Reception-SW(config-if-range)#switchport port-security violation shutdown

Reception-SW(config-if-range)#ex

Reception-SW(config)#do wr

Building configuration...

[OK]

Reception-SW(config)#

Industrial Devices CLI Commands: -

ID-SW(config)#int range fa0/4-24

ID-SW(config-if-range)#switchport port-security maximum 1

ID-SW(config-if-range)#switchport port-security mac-address sticky

ID-SW(config-if-range)#switchport port-security violation shutdown

ID-SW(config-if-range)#ex

ID-SW(config)#do wr

Building configuration...

[OK]

ID-SW(config)#