Problem Statement for Minor Project:

Develop a scalable network for a manufacturing plant, integrating different units like Assembly, Quality Control, Logistics, and Administration. The network must support:

• Seamless communication between production lines and inventory systems.

• VLAN segmentation for secure communication between IoT devices, SCADA systems, and enterprise networks.

• Real-time monitoring to detect and respond to network issues without disrupting operations.

Challenge:

Ensure high availability and protection against industrial cyber threats targeting OT systems.

Scenario:

Manufacturing Plant with different zones likes Production Zone, Engineering Zone, Corporate Zone, Storage Zone, DMZ (Demilitarized Zone), Guest & Miscellaneous Areas with VLAN segmentation, IoT devices, SCADA systems.

Scenario Overview: Designing and implementing a comprehensive network for a manufacturing plant which deals with both manufacturing and trading. The plant has five zones, with multiple departments distributed across in each zone. The network must ensure seamless wired and wireless connectivity, VLAN segmentation for secure communication between IoT devices, SCADA systems, and enterprise networks, real-time monitoring to detect and respond to network issues without disrupting operations, ensuring high availability and protection against industrial cyber threats targeting OT systems. Additionally, the network should be future-proofed to handle scalability.

Zone Layout:

- Production Zone: Includes industrial devices such as Programmable

Controllers (PLCs), sensors, and actuators that control and monitor production

lines.

- Engineering Zone: Houses engineering workstations and servers used for desicung and maintaining production systems.

- Corporate Zone: Supports administrative systems for HR, finance, and general operations.

- Storage Zone: Contains servers for storing production data, inventory management systems, and backups.

- Guest & Miscellaneous Areas (Separate VLANs within Corporate Zone for Limited Access): Contains WAP for guests and outside visitors.

- DMZ (Demilitarized Zone): Hosts public-facing services such as vendor portals and communication systems.

This segmentation enhances performance, ensures security, and supports scalability

while protecting critical systems from external threats.

Department Distribution:

- Production Zone:

  - Industrial devices (PLCs, sensors, actuators)

  - Manufacturing production lines

- Engineering Zone:

  - Engineering workstations

  - Design and maintenance servers

- Corporate Zone:

  - Sales and Marketing

  - Human Resources

  - Finance and Accounts

  - Administrator and Public Relations

  - IT Department

- Storage Zone:

  - Inventory management systems

  - Production data servers

  - Backup storage

  - Logistics

- DMZ (Demilitarized Zone):

  - Vendor Portals & Communication Systems

  - Security Department

  - Server Room

- Guest & Miscellaneous Areas

  - Reception and Visitor Lounge

  - Cafeteria

Network Design:

Hierarchical Network Design Mapping

A hierarchical network is structured into three layers:

1. Core Layer (High-Speed Backbone)

- Connects all major zones with high-speed switching and redundancy.

- Typically includes high-performance routers and core switches.

- Provides fast, resilient connectivity between data centers, corporate networks, and production systems.

- Zones Connected:

  - Production Zone

  - Engineering Zone

  - Corporate Zone

  - Storage Zone

  - DMZ

2. Distribution Layer (Policy Enforcement & Security)

- Aggregates connections from different zones.

- Implements VLANs, routing, firewall rules, and network access control to enforce security.

- Manages Quality of Service (QoS) for traffic prioritization (e.g., real-time production data).

- Departments Managed:

  - Security controls for Production & Engineering Zones (OT Security)

  - Secure segmentation of Corporate IT & Storage Zone

  - Controlled access for DMZ services

3. Access Layer (Edge Devices & User Connectivity)

- Provides connectivity for end devices such as workstations, IoT devices, and wireless access points.

- Implements Access Control Lists (ACLs), VLAN tagging, and security policies to restrict unauthorized access.

- Devices Connected:

  - Production Zone: PLCs, sensors, industrial controllers

  - Engineering Zone: Workstations, CAD/PLM servers

  - Corporate Zone: Employee workstations, office networks

  - Storage Zone: Inventory systems, database servers

  - DMZ: External vendor portals, firewalls

VLANs and IP Addressing:

- Each department should be in a separate VLAN and subnet to segregate traffic and enhance security.

- Use the base network 172.16.0.0/16 for subnetting and allocate IP addresses dynamically (DHCP) for devices in each department.

- Static IPs will be assigned for servers and critical devices in the Server Room.

### . Multilayer Switches ↔ Firewalls (/30 = 255.255.255.252)

| Link | Subnet | Subnet Mask | ML Switch IP | Firewall IP |
| --- | --- | --- | --- | --- |
| MLS1 ↔ FW1 | 172.16.240.0/30 | 255.255.255.252 | 172.16.240.1 | 172.16.240.2 |
| MLS1 ↔ FW2 | 172.16.240.4/30 | 255.255.255.252 | 172.16.240.5 | 172.16.240.6 |
| MLS1 ↔ FW3 | 172.16.240.8/30 | 255.255.255.252 | 172.16.240.9 | 172.16.240.10 |
| MLS2 ↔ FW1 | 172.16.240.8/30 | 255.255.255.252 | 172.16.240.9 | 172.16.240.10 |
| MLS2 ↔ FW2 | 172.16.240.12/30 | 255.255.255.252 | 172.16.240.13 | 172.16.240.14 |
| MLS2 ↔ FW3 | 172.16.240.28/30 | 255.255.255.252 | 172.16.240.29 | 172.16.240.30 |
| MLS3 ↔ FW1 | 172.16.240.16/30 | 255.255.255.252 | 172.16.240.17 | 172.16.240.18 |
| MLS3 ↔ FW3 | 172.16.240.32/30 | 255.255.255.252 | 172.16.240.30 | 172.16.240.29 |
| MLS3 ↔ FW2 | 172.16.240.20/30 | 255.255.255.252 | 172.16.240.21 | 172.16.240.22 |

### 🔁 2. Firewalls ↔ Routers (/30 = 255.255.255.252)

| Link | Subnet | Subnet Mask | Firewall IP | Router IP |
| --- | --- | --- | --- | --- |
| FW1 ↔ R1 | 172.16.240.24/30 | 255.255.255.252 | 172.16.240.25 | 172.16.240.26 |
| FW1 ↔ R2 | 172.16.240.28/30 | 255.255.255.252 | 172.16.240.29 | 172.16.240.30 |
| FW2 ↔ R1 | 172.16.240.32/30 | 255.255.255.252 | 172.16.240.33 | 172.16.240.34 |
| FW2 ↔ R2 | 172.16.240.36/30 | 255.255.255.252 | 172.16.240.37 | 172.16.240.38 |

### ☁️ 3. Routers ↔ Cloud (/30 = 255.255.255.252)

| Link | Subnet | Subnet Mask | Router IP | Cloud Gateway IP |
| --- | --- | --- | --- | --- |
| R1 ↔ Cloud | 172.16.240.40/30 | 255.255.255.252 | 172.16.240.41 | 172.16.240.42 |
| R2 ↔ Cloud | 172.16.240.44/30 | 255.255.255.252 | 172.16.240.45 | 172.16.240.46 |

Each /30 subnet has:

- Network address (first IP, e.g., 172.16.240.0)

- 2 usable host IPs (e.g., .1 and .2)

- Broadcast address (last IP, e.g., .3)

- VLAN and Subnet Allocation:

- VLAN Assignment and Subnet Mapping

| Zone | Department | VLAN ID | Subnet | Subnet Mask | IP Range |
| --- | --- | --- | --- | --- | --- |
| Production Zone | Industrial Devices (PLCs, Sensors, Actuators) | 60 | 172.16.0.2/20 | 255.255.240.0 | 172.16.0.1 - 172.16.15.254 |
|  | Manufacturing Production Lines | 11 | 172.16.16.2/20 | 255.255.240.0 | 172.16.16.1 - 172.16.31.254 |
| Engineering Zone | Engineering Workstations | 20 | 172.16.64.2/19 | 255.255.224.0 | 172.16.64.1 - 172.16.95.254 |
|  | Design & Maintenance Servers | 21 | 172.16.96.2/19 | 255.255.224.0 | 172.16.96.1 - 172.16.127.254 |
| Corporate Zone | Sales & Marketing | 30 | 172.16.128.2/20 | 255.255.240.0 | 172.16.128.1 - 172.16.143.254 |
|  | Human Resources | 31 | 172.16.144.2/21 | 255.255.248.0 | 172.16.144.1 - 172.16.151.254 |
|  | Finance & Accounts | 32 | 172.16.152.2/21 | 255.255.248.0 | 172.16.152.1 - 172.16.159.254 |
|  | Administrator & PR | 33 | 172.16.160.2/21 | 255.255.248.0 | 172.16.160.1 - 172.16.167.254 |
|  | IT Department | 34 | 172.16.168.2/21 | 255.255.248.0 | 172.16.168.1 - 172.16.175.254 |
| Storage Zone | Inventory Management | 40 | 172.16.192.2/22 | 255.255.252.0 | 172.16.192.1 - 172.16.195.254 |
|  | Backup Storage | 42 | 172.16.200.2/22 | 255.255.252.0 | 172.16.200.1 - 172.16.203.254 |
|  | Logistics | 43 | 172.16.204.2/22 | 255.255.252.0 | 172.16.204.1 - 172.16.207.254 |
| DMZ Zone | Vendor Portals & Security | 50 | 172.16.208.2/24 | 255.255.255.0 | 172.16.208.1 - 172.16.208.254 |
|  | Server Room (Static IPs) | 51 | 172.16.209.2/25 | 255.255.255.128 | 172.16.209.1 - 172.16.209.126 |
|  | Security Department | 52 | 172.16.210.2/25 | 255.255.255.128 | 172.16.210.1 - 172.16.210.126 |
| Guest & Public Access | Cafeteria WiFi (Guests Only) | 10 | 172.16.216.2/24 | 255.255.255.0 | 172.16.216.1 - 172.16.216.254 |
|  | Reception & Visitor Lounge | 61 | 172.16.217.2 /24 | 255.255.255.0 | 172.16.217.1 - 172.16.217.254 |

Static IP Assignments for Critical Devices in Server Room:

| Device | Static IP |
| --- | --- |
| Main Firewall | 172.16.209.1 |
| Primary Database Server | 172.16.209.2 |
| Secondary Database Server | 172.16.209.3 |
| Web Server | 172.16.209.4 |
| Backup Server | 172.16.209.5 |
| Active Directory Server | 172.16.209.6 |
| Network Switch 1 | 172.16.209.10 |
| Network Switch 2 | 172.16.209.11 |
| Network Printer | 172.16.209.20 |

Routing:

- Use OSPF as the routing protocol to advertise internal routes across the network.

- Configure NAT Overload (PAT) on the core routers for internet access.

Wireless Connectivity:

- Deploy one Cisco Access Point (AP) per department for mobile devices, smartphones, and laptops to connect wirelessly.

- Use SSIDs to each department (e.g., "Sales_WiFi," "HR_WiFi") with WPA3 encryption for security.specific

- Set up a Guest WiFi network isolated from the corporate network.

DHCP and IP Allocation:

- Configure a dedicated DHCP server in the Server Room to dynamically allocate IPs to all devices (except those in the server room, which use static IPs).

- Each router will act as a DHCP relay for devices on its floor.

Redundancy:

- Use two core routers connected to two ISPs with static public IPs (195.136.17.0/30 and 195.136.17.4/30) for internet redundancy.

- Use HSRP (Hot Standby Router Protocol) or similar for failover between core routers.

- Configure redundant links between distribution and access layers.

Security:

- Deploy a firewall in the Server Room to control incoming and outgoing traffic.

- Enable port security on all access switches to prevent unauthorized devices.

  - For all other zones except DMZ, allow only one device per switchport using the sticky MAC method, with violation mode set to "shutdown."

- Configure SSH on all routers and multilayer switches for secure remote management.

- Use ACLs to restrict access to sensitive resources.

Testing and Monitoring:

- Add a Test-PC in the IT department to test connectivity and remote login via SSH.

- Configure SNMP and Syslog for network monitoring.

- Test inter-VLAN communication, internet access, and failover mechanisms.

Future-Proofing:

- Reserve unused VLANs for future departments or expansions.

- Implement scalable cabling infrastructure (Cat6 or fiber optic) to support higher speeds.

- Ensure devices support IPv6 for future adoption.

Technologies and Tools Used:

- Cisco Packet Tracer for simulation and implementation.

- Hierarchical Network Design for scalability and redundancy.

- VLANs and Inter-VLAN Routing for traffic segregation and communication.

- OSPF Routing for dynamic route advertisement.

- Port Address Translation (PAT) for internet access.

- Port Security to restrict unauthorized access.

- SSH and ACLs for secure management and resource control.

- DHCP Server for dynamic IP allocation.

- Wireless Access Points for seamless WiFi connectivity.

- SCADA Network Security to protect industrial control systems from cyber threats.

- Real-time Network Monitoring (SNMP, Syslog, NetFlow) to detect and respond to issues proactively.

- Industrial Firewalls & Intrusion Prevention Systems (IPS) to safeguard OT (Operational Technology) networks.

- Redundant Links & Failover Mechanisms (HSRP, VRRP) to ensure high availability.

- IoT Device Segmentation for secure communication between sensors, actuators, and enterprise systems.

- Network Time Protocol (NTP) for synchronized time across all network devices.

- Zero Trust Security Model to enforce strict access controls for critical systems.

- Multi-Factor Authentication (MFA) & Role-Based Access Control (RBAC) for network security.