Scenario:  
Multi-Floor Corporate Building with Integrated Wireless, Security, and Redundancy

Scenario Overview: Designing and implementing a comprehensive network for a corporate building hosting a company that deals in both manufacturing and trading. The building has 4 floors, with multiple departments distributed across them. The network must ensure seamless wired and wireless connectivity, redundancy at critical layers, inter-departmental communication, and secure internet access. Additionally, the network should be future-proofed to handle scalability and provide high availability.

Building Layout and Department Distribution:

- Ground Floor:

  - Reception and Visitor Lounge (30 devices)

  - Security Department (20 devices)

  - Cafeteria (WiFi access only for 50 devices)

- First Floor:

  - Sales and Marketing (120 devices)

  - Human Resources (60 devices)

  - Logistics (60 devices)

- Second Floor:

  - Finance and Accounts (120 devices)

  - Administrator and Public Relations (120 devices)

- Third Floor:

  - IT Department (120 devices)

  - Server Room (20 static devices, including a firewall)

Business Requirements and Network Design:

1. Logical Design:

- Use a hierarchical network model with three layers:

  - Core Layer: Two routers connected to two ISPs for redundancy.

  - Distribution Layer: Two multilayer switches for inter-VLAN routing and redundancy.

  - Access Layer: Dedicated access switches on each floor for end-user devices and wireless Access Points (APs).

2. VLANs and IP Addressing:

- Each department should be in a separate VLAN and subnet to segregate traffic and enhance security.

- Use the base network 172.16.0.0/16 for subnetting and allocate IP addresses dynamically (DHCP) for devices in each department.

- Static IPs will be assigned for servers and critical devices in the Server Room.

VLAN and Subnet Allocation:

| Department/Area | VLAN ID | Subnet | Devices Expected |
| --- | --- | --- | --- |
| Reception & Visitor Lounge | 100 | 172.16.1.0/24 | 30 |
| Security | 110 | 172.16.2.0/24 | 20 |
| Cafeteria (WiFi only) | 120 | 172.16.3.0/24 | 50 |
| Sales & Marketing | 130 | 172.16.4.0/24 | 120 |
| Human Resources | 140 | 172.16.5.0/24 | 60 |
| Logistics | 150 | 172.16.6.0/24 | 60 |
| Finance & Accounts | 160 | 172.16.7.0/24 | 120 |
| Administrator & PR | 170 | 172.16.8.0/24 | 120 |
| IT Department | 180 | 172.16.9.0/24 | 120 |
| Server Room | 190 | 172.16.10.0/24 | 20 (static) |

3. Routing:

- Use OSPF as the routing protocol to advertise internal routes across the network.

- Configure NAT Overload (PAT) on the core routers for internet access.

4. Wireless Connectivity:

- Deploy one Cisco Access Point (AP) per department for mobile devices, smartphones, and laptops to connect wirelessly.

- Use SSIDs to each department (e.g., "Sales_WiFi," "HR_WiFi") with WPA3 encryption for security.specific

- Set up a Guest WiFi network isolated from the corporate network.

5. DHCP and IP Allocation:

- Configure a dedicated DHCP server in the Server Room to dynamically allocate IPs to all devices (except those in the server room, which use static IPs).

- Each router will act as a DHCP relay for devices on its floor.

6. Redundancy:

- Use two core routers connected to two ISPs with static public IPs (195.136.17.0/30 and 195.136.17.4/30) for internet redundancy.

- Use HSRP (Hot Standby Router Protocol) or similar for failover between core routers.

- Configure redundant links between distribution and access layers.

7. Security:

- Deploy a firewall in the Server Room to control incoming and outgoing traffic.

- Enable port security on all access switches to prevent unauthorized devices.

  - For the Finance department, allow only one device per switchport using the sticky MAC method, with violation mode set to "shutdown."

- Configure SSH on all routers and multilayer switches for secure remote management.

- Use ACLs to restrict access to sensitive resources.

8. Testing and Monitoring:

- Add a Test-PC in the IT department to test connectivity and remote login via SSH.

- Configure SNMP and Syslog for network monitoring.

- Test inter-VLAN communication, internet access, and failover mechanisms.

9. Future-Proofing:

- Reserve unused VLANs for future departments or expansions.

- Implement scalable cabling infrastructure (Cat6 or fiber optic) to support higher speeds.

- Ensure devices support IPv6 for future adoption.

Technologies and Tools Used:

- Cisco Packet Tracer for simulation and implementation.

- Hierarchical Network Design for scalability and redundancy.

- VLANs and Inter-VLAN Routing for traffic segregation and communication.

- OSPF Routing for dynamic route advertisement.

- Port Address Translation (PAT) for internet access.

- Port Security to restrict unauthorized access.

- SSH and ACLs for secure management and resource control.

- DHCP Server for dynamic IP allocation.

- Wireless Access Points for seamless WiFi connectivity.