Synopsis Report: Manufacturing Plant Network

Project Title: Design and Implementation of a Secure and Scalable Network Infrastructure for a Manufacturing Plant.

Group Number: 03

Project Team:

- Amal Antony Alex – FET-BCD-2023-27-020

- Swosti Sanhita Jena - FET-BCD-2023-27-022

- Chidananda Sashank Bhuyan - FET-BCD-2023-27-025

- Chiranjibi Dash - FET-BCD-2023-27-026

Mentors:

- Internal Mentor: Dr. Chinmaya Kumar Nayak

- External Mentor: Ms. Neha Bagle

- Faculty: Faculty of Engineering & Technology, Sri Sri University, Cuttack

1. Abstract

This project focuses on designing a hierarchical, scalable, and secure network for a manufacturing plant. The proposed network follows a three-layer model (Core, Distribution, Access) to ensure high availability and performance. The segmentation of the plant into Production, Engineering, Corporate, Storage, DMZ, and Guest zones is achieved using VLANs. Critical security measures such as firewalls, intrusion prevention, and access control lists (ACLs) protect against cyber threats targeting industrial control systems (OT). The objective is to create a resilient network that facilitates seamless data flow and real-time communication.

2. Introduction

- Domain: Scalable network design for manufacturing plants.

- Problem Statement: Securely integrating diverse units (Assembly, QC, Logistics, Admin) with real-time monitoring.

- Importance: Ensures operational efficiency, cybersecurity, and future scalability.

- Approach:

  - Hierarchical Network Design: Core, Distribution, and Access layers.

  - Security Measures: VLAN segmentation, firewalls, and OT security.

3. Literature Review

- Studies on hierarchical network design in industrial environments.

- Use of VLANs for segmentation in secure industrial networks.

- Importance of cybersecurity in OT and IoT-based networks.

4. Objectives

- Develop a comprehensive network for the manufacturing plant.

- Implement VLANs for secure communication.

- Enable real-time monitoring to detect and resolve issues.

- Ensure high availability with redundancy mechanisms.

- Protect OT systems from industrial cyber threats.

5. Motivation

- Interconnected systems are essential for manufacturing efficiency.

- Increasing cyber threats necessitate strong security measures.

- Scalability is key for accommodating future growth.

- Provides hands-on experience in network design and security.

6. Problem Statement

Develop a scalable and secure network for a manufacturing plant integrating:

- Assembly, Quality Control, Logistics, and Administration.

- VLAN segmentation for IoT devices, SCADA systems, and enterprise networks.

- Real-time monitoring and protection against cyber threats in OT systems.

7. Work Plan & Timeline

| Phase | Tasks | Duration |
| --- | --- | --- |
| Phase 1 | Research & Literature Review | 2-3 Weeks |
| Phase 2 | Network Design & Planning | 2-3 Weeks |
| Phase 3 | VLAN, IP Configuration, Routing & Security | 4-5 Weeks |
| Phase 4 | Testing & Documentation | 2-3 Weeks |
| Phase 5 | Final Submission | March 31, 2025 |

8. Conclusion

This project presents a real-world network design solution that enhances security, scalability, and efficiency in a manufacturing plant. VLAN segmentation, routing protocols, and security measures improve performance while real-time monitoring minimizes downtime. The design is future-ready, supporting technological advancements.

9. Future Scope

- AI-driven network analytics for failure prediction and bandwidth optimization.

- 5G-enabled IoT devices for ultra-low latency communication.

- IPv6 adoption for long-term scalability.

- Blockchain authentication for secure vendor interactions.

- Biometric-based network access control (NAC) for enhanced security.

- Cloud-native SD-WAN for dynamic traffic management.

- Digital twin technology for network simulation and optimization.

10. References

- Mwansa, G., Ngandu, M. R., & Dasi, Z. S. (2024). Enhancing Practical Skills in Computer Networking: Evaluating the Unique Impact of Simulation Tools, Particularly Cisco Packet Tracer, in Resource-Constrained Higher Education Settings. Education Sciences, 14(10), 1099.

- Das, B. J., Chidambaram, V., & Palanidoss, S. (2023, November). Packet tracer for smart home networks and real-world monitoring. In AIP Conference Proceedings (Vol. 2946, No. 1). AIP Publishing.

- Malanchini, I., Michailow, N., Agostini, P., Ali-Tolppa, J., Hock, D., Kasparick, M., ... & Zhou, Q. (2023). Convergence of Manufacturing and Networking in Future Factories. arXiv preprint arXiv:2312.08708.

- Lindenschmitt, D., Mertes, J., Schellenberger, C., Schmitz, M., Han, B., Aurich, J. C., & Schotten, H. D. (2023, October). 6G Underlayer Network Concepts for Ultra Reliable and Low Latency Communication in Manufacturing. In European Wireless 2023; 28th European Wireless Conference (pp. 26-30). VDE.

- Allison, J. (2022, July). Simulation-based learning via cisco packet tracer to enhance the teaching of computer networks. In Proceedings of the 27th ACM Conference on on Innovation and Technology in Computer Science Education Vol. 1 (pp. 68-74).

- Cisco Networking Cisco Networking Academy. (2020). Switching, Routing, and Wireless Essentials Companion Guide (CCNAv7). Pearson Education, Limited.

- Kurose, J. F., & Ross, K. W. (2007). Computer networking: A top-down approach edition. Addision Wesley.

- Coleman, D. D., & Westcott, D. A. (2014). CWNA: Certified wireless network administrator official study guide: Exam CWNA-106. John Wiley & Sons.