Testing Scenario:

1. Inter-VLAN Communication

Objective: Ensure that devices in different VLANs can communicate where allowed.

Test Steps:

- From the Test-PC in the IT Department (VLAN 34), ping:

  - A PC in Sales (VLAN 30) – Expected: Success (allowed inter-departmental communication).

  - A PLC in Production Zone (VLAN 60) – Expected: Fail (restricted by ACLs).

  - The Primary Database Server (Static IP 172.16.209.2) – Expected: Success.

Expected Result:

- Pings to VLANs allowed by ACLs succeed.

- Pings to restricted VLANs fail, confirming ACLs are functioning.

2. Internet Connectivity via NAT (PAT)

Objective: Validate that internal clients can access the internet through NAT.

Test Steps:

- From the Guest WiFi client (VLAN 10) and HR Workstation (VLAN 31), try to access an external server (simulate using a cloud DNS or web server).

Expected Result:

- Internet access is successful through NAT configured on core routers (R1/R2).

- HSRP failover maintains connectivity if one router goes down.

3. DHCP Operation

Objective: Verify that devices in each department receive correct IPs.

Test Steps:

- Connect a new PC in VLAN 33 (PR Dept).

- Monitor IP assignment from the centralized DHCP server.

Expected Result:

- Device receives an IP within the 172.16.160.1–172.16.167.254 range.

- Gateway and DNS configurations are correct.

4. VLAN Segmentation & Security Policies

Objective: Ensure network isolation as per the design.

Test Steps:

- Try pinging between:

  - Sales (VLAN 30) and Finance (VLAN 32) – Expected: Fail.

  - Engineering Server (VLAN 21) to Design Workstation (VLAN 20) – Expected: Success.

Expected Result:

- Communication is restricted between business units unless explicitly allowed.

- Engineering zone allows intra-zone communication.

5. Redundancy and Failover (HSRP + Redundant Links)

Objective: Test high availability in case of router failure.

Test Steps:

- Shut down Router R1.

- Monitor internet connectivity from IT workstation.

Expected Result:

- Traffic reroutes through Router R2 using HSRP.

- No downtime observed for end users.

6. Wireless Connectivity

Objective: Confirm proper SSID-based wireless access.

Test Steps:

- Connect a wireless laptop to "Sales_WiFi" and access internal resources.

- Connect another device to "Guest_WiFi" and attempt access to internal VLANs.

Expected Result:

- Sales_WiFi user has internal access.

- Guest_WiFi user is isolated (access only to the internet).

7. Remote Management via SSH

Objective: Validate secure management access.

Test Steps:

- From Test-PC, SSH into core switch and router using configured credentials.

Expected Result:

- SSH session is established securely.

- Device configuration is accessible only with proper login.

8. Monitoring and Logging (SNMP/Syslog)

Objective: Ensure real-time monitoring is operational.

Test Steps:

- Force a port security violation by plugging in a rogue device to an access port.

- Observe if alert is generated in the Syslog server.

Expected Result:

- Port enters shutdown state.

- SNMP trap or Syslog message is logged confirming the violation.

Summary of Results

| Test Case | Expected Outcome | Status |
| --- | --- | --- |
| Inter-VLAN Communication | Allowed where configured | Success |
| NAT Internet Access | Works via core routers | Success |
| DHCP Allocation | Dynamic IPs assigned correctly | Success |
| VLAN Security | Isolation maintained | Success |
| Redundancy | No downtime during failover | Success |
| Wireless SSIDs | Secure access & guest isolation | Success |
| SSH Management | Secure remote access works | Success |
| Port Security & Logging | Unauthorized device blocked & logged | Success |